🛠️ Bearing RUL Diagnostic System

Uncertainty-Aware Bearing Remaining Useful Life Prediction with Maintenance Scheduling

An AI-based predictive maintenance system designed to estimate the Remaining Useful Life (RUL) of rolling-element bearings using vibration data. The system combines machine learning, uncertainty estimation, SHAP explainability, maintenance decision policies, and AI-generated maintenance explanations.

📌 Project Overview

Unexpected bearing failures can cause production downtime, increased maintenance costs, and equipment damage. Traditional maintenance strategies such as reactive maintenance and fixed time-based replacement may either replace healthy components too early or fail to prevent unexpected failures.

This project addresses this problem by developing an Uncertainty-Aware Bearing Remaining Useful Life Prediction System.

The system analyzes bearing vibration signals, extracts meaningful features, predicts the remaining useful life of the bearing, estimates prediction uncertainty, and recommends appropriate maintenance actions.

The application also provides an explainability module using SHAP, helping users understand which vibration features influence the RUL prediction.

🎯 Objectives

The main objectives of this project are:

- Predict the Remaining Useful Life (RUL) of bearings.

- Extract meaningful features from vibration signals.

- Compare machine learning models for RUL prediction.

- Estimate uncertainty in predictions using prediction intervals.

- Evaluate models using appropriate performance metrics.

- Provide maintenance recommendations based on predicted RUL and uncertainty.

- Explain model predictions using SHAP feature importance.

- Provide AI-assisted maintenance explanations.

- Develop an interactive diagnostic dashboard using Streamlit.

⚙️ System Architecture

The project follows the following workflow:

📊 Key Features

🔹 RUL Prediction

The system predicts the Remaining Useful Life of a bearing based on extracted vibration features.

🔹 Feature Engineering

Features are extracted from horizontal and vertical vibration signals.

Examples include:

- Mean

- Standard Deviation

- Variance

- RMS

- Skewness

- Kurtosis

- Peak Values

- Shape Factor

- Frequency Band Energy

- Spectral Features

🔹 Model Comparison

Multiple machine learning models can be evaluated to identify the best-performing model.

Models include:

- XGBoost

- LightGBM

- Random Forest

- CatBoost

Performance results are stored for comparison.

🔹 Uncertainty Estimation

Instead of providing only a single RUL prediction, the system estimates a prediction interval.

For example:

Predicted RUL: 74.8%

Prediction Interval: 9.4% – 98.1%

This provides additional information about the confidence and reliability of the prediction.

🔹 SHAP Explainability

SHAP analysis is used to identify the features that have the greatest influence on RUL predictions.

Example important features:

horiz_band_energy_2

horiz_band_energy_5

horiz_std

horiz_shape_factor

horiz_variance

horiz_peak

This helps make the machine learning predictions more interpretable.

🔹 Risk-Aware Maintenance Policy

The system uses predicted RUL and uncertainty information to recommend maintenance actions.

Possible actions include:

- Continue Operation

- Monitor Closely

- Schedule Maintenance

- Inspect Before Replacement

- Immediate Maintenance

The recommendations help support predictive maintenance decision-making.

🔹 AI-Assisted Maintenance Explanation

The explainability section generates a plain-language maintenance explanation based on:

- Selected bearing

- Current snapshot

- Predicted RUL

- Prediction interval

- SHAP feature importance

- Maintenance recommendation

Example explanation areas:

- Bearing Health Summary

- RUL Interpretation

- Uncertainty Interpretation

- SHAP Feature Insights

- Maintenance Recommendation

The AI explanation is grounded using the prediction values already generated by the system.

🖥️ Streamlit Dashboard

The application provides an interactive diagnostic and maintenance dashboard with the following sections:

1. Overview

Provides a summary of the project and system status.

2. RUL Prediction

Displays:

- Bearing selection

- Snapshot selection

- Predicted Remaining Useful Life

3. Model Performance

Displays model evaluation and comparison results.

4. Uncertainty

Displays uncertainty metrics and prediction intervals.

5. Maintenance Policy

Provides maintenance recommendations based on prediction and uncertainty.

6. Explainability

Displays:

- SHAP feature importance

- Important prediction drivers

- AI-generated maintenance explanation

📁 Dataset

This project uses bearing vibration data for Remaining Useful Life prediction.

The dataset contains vibration signals collected from bearings under different operating conditions and degradation stages.

The vibration signals are processed to extract statistical, time-domain, and frequency-domain features used for machine learning-based RUL prediction.

📂 Project Structure

🧠 Machine Learning Pipeline

Step 1: Data Collection

Bearing vibration data is loaded from the available dataset.

Step 2: Data Preprocessing

The data is cleaned and prepared for analysis.

Step 3: Feature Extraction

Statistical and frequency-based features are extracted from vibration signals.

Step 4: Model Training

Machine learning models are trained using the extracted features.

Step 5: Model Evaluation

Models are evaluated and compared based on prediction performance.

Step 6: Uncertainty Estimation

Prediction intervals and uncertainty metrics are calculated.

Step 7: SHAP Analysis

SHAP values are used to determine the importance of individual features.

Step 8: Maintenance Decision

A risk-aware policy generates maintenance recommendations.

Step 9: Dashboard Visualization

Results are presented through an interactive Streamlit application.

📈 Evaluation Metrics

The system evaluates prediction performance using metrics such as:

- Mean Absolute Error (MAE)

- Model comparison metrics

- Prediction Interval Coverage Probability (PICP)

- Uncertainty-related metrics

These metrics help evaluate both the accuracy and reliability of the RUL prediction system.

🚀 Installation

1. Clone the Repository

git clone https://github.com/Koushal06/Bearing-RUL-Diagnostic-System.git

2. Navigate to the Project Folder

cd Bearing-RUL-Diagnostic-System

3. Create a Virtual Environment

python -m venv venv

4. Activate the Virtual Environment

Windows

venv\Scripts\activate

Linux / macOS

source venv/bin/activate

5. Install Dependencies

pip install -r requirements.txt

▶️ Running the Application

Run the Streamlit application using:

streamlit run app/app.py

After running the command, open:

http://localhost:8501

The Bearing RUL Diagnostic and Maintenance Console will open in your browser.

🛡️ Maintenance Decision Logic

The maintenance policy considers both:

- Predicted Remaining Useful Life

- Prediction Uncertainty

This is important because a prediction alone may not be sufficient for a reliable maintenance decision.

For example:

This approach supports more informed and risk-aware maintenance decisions.

🔍 Explainability Example

The system identifies the features that contribute most to a prediction.

Example:

Top Driving Feature:

horiz_band_energy_2

Other Important Features:

horiz_band_energy_5

horiz_std

horiz_shape_factor

horiz_variance

SHAP values help answer:

Why did the model predict this RUL value?

This improves the transparency and interpretability of the predictive maintenance system.

💻 Technologies Used

Programming Language

- Python

Machine Learning

- XGBoost

- CatBoost

- Scikit-learn

Explainable AI

- SHAP

Data Processing

- NumPy

- Pandas

- SciPy

Visualization

- Matplotlib

- Plotly

Web Application

- Streamlit

Environment Management

- Python Dotenv

Version Control

- Git

- GitHub

🎓 Academic Project

This project was developed as an academic project focused on:

Uncertainty-Aware Bearing Remaining Useful Life (RUL) Prediction with Cost-Based / Risk-Aware Maintenance Scheduling

The project combines concepts from:

- Machine Learning

- Predictive Maintenance

- Remaining Useful Life Prediction

- Uncertainty Quantification

- Explainable Artificial Intelligence

- Decision Support Systems

🔮 Future Enhancements

Possible future improvements include:

- Real-time sensor data integration.

- IoT-based vibration monitoring.

- Deep learning models such as LSTM and CNN.

- Advanced uncertainty estimation techniques.

- Automated maintenance alerts.

- Cloud deployment.

- Database integration.

- Multi-bearing fleet monitoring.

- Maintenance cost optimization.

- Enhanced AI-generated maintenance reports.

👨‍💻 Author

Koushal V

GitHub: Koushal06

⭐ Conclusion

The Bearing RUL Diagnostic System provides an end-to-end framework for predicting bearing degradation and supporting maintenance decisions. By combining RUL prediction, uncertainty estimation, SHAP explainability, and risk-aware maintenance recommendations, the system provides more than just a prediction—it delivers an interpretable and actionable diagnostic solution.